To Run WSJT-X Within Station Master

Install WSJT-X and getting CAT Control working

(skip step 5 & 6 if you have WSJT-X installed)

- Open Station Master and from the top menu’s start rig control then go to FILE/SETTINGS in station master then go to the 3RD PARTY APPS tab.

- In 3rd Party Apps Tab on the right-hand side enable option for “start a HAMLIB RigCtld Server”.

- Click “Save and Restart” and let Station Master Restart.

- When Station Master Restarts then navigate to the Launcher menu at the top.

- Select WSJT-X in the launcher menu and it will say it is not installed do you want to download it.

- Say yes to downloading WSJT-X, again it will download and install.

- Open WSJT-X via the launcher menu.

- In WSJT-X go to FILE/SETTINGS.

- Navigate to “Reporting” then to “Secondary UDP Server” at the bottom

- Check “enable logged contact ADIF broadcast” and set “Server name or IP address” to 127.0.0.1 (if not already done so for you) & set port number to 2333 (if not already done for you).

- Navigate to the “RADIO” tab and in the “RIG” selection select “Hamlib NET rigctl”.

- In the “RADIO” tab under “PTT Method” select your PTT Method.

- In the “RADIO” tab under “Mode” select your mode.

- In the “RADIO” tab under “Split Operation” select your Operation Type.

- Click test CAT and it should connect.